Reloading The .308 Winchester Rifle 


Loads for .308 Winchester Rifle:

(Note: These are near maximum loads, you should use 10% less to start.)
(See note on Powders below or read all about various Powders.) 
Bullet is .308" diameter. Due to different barrel lengths, type of bullet, seating depth, primer type and other factors, you may not get near the FPS charted. It is just a guide and the reason you should start under these charges and work up. 

----------------------------------------

100 grain Plinker (Cup Jacket with lead nose)

    IMR 4895     45.0 gr.   2,770 FPS

    IMR 3031     44.0 gr.   3,080

    Reloader 11  43.0 gr.   2,900

-----------------------------------------

110 grain JSP or JHP (Sierra Jacketed Spire Point)

    IMR 4895     49.0*gr.   3,130 FPS

    H4895        49.0*gr.   3,239

    IMR 3031     45.0 gr.   2,990

    Reloader 7   42.5 gr.   3,130

    Reloader 12  50.5 gr.   3,200

    H380         53.0 gr.   3,145

    A2230        47.5 gr.   3,171

    A2460        48.5 gr.   3,193

    748          53.2 gr.   3,300

* compressed load

-----------------------------------------

125 grain JSP (Sierra Spitzer Point)

    IMR 4895     49.0 gr.   2,925 FPS

    H4895        47.0 gr.   3,075

    Reloader 7   40.0 gr.   2,920

    Reloader 12  49.0 gr.   3,040

    H380         52.0 gr.   2,986

    A2230        47.0 gr.   3,017

    A2460        47.0 gr.   3,014

    748          52.0 gr.   3,175

-----------------------------------------

150 grain JSP (Spitzer or Spire Point)

    IMR 4895     44.5 gr.   2,780 FPS

    H4895        44.0 gr.   2,830

    IMR 3031     45.0*gr.   2,830

    2400         25.0 gr.   2,215

    Reloader 7   37.0 gr.   2,750

    Reloader 15  46.3 gr.   2,880

    H380         51.0 gr.   2,876

    A2230        43.5 gr.   2,712

    A2460        45.0 gr.   2,765

    748          48.5 gr.   2,865

* compressed load

-----------------------------------------

165 grain JSP (Spitzer or Spire Point)

    IMR 4895     41.0 gr.   2,421 FPS

    H4895        40.0 gr.   2,525

    IMR 3031     37.0 gr.   2,457

    Reloader 12  44.0 gr.   2,650

    Reloader 15  45.5 gr.   2,780

    H380         48.0 gr.   2,634

    A2230        42.0 gr.   2,610

    A2460        42.5 gr.   2,601

-----------------------------------------

180 grain JSP (Spitzer or Spire Point) or JRN

    IMR 4895     41.0 gr.   2,346 FPS

    H4895        40.0 gr.   2,540

    IMR 3031     38.0 gr.   2,411

    2400         28.2 gr.   2,210

    Reloader 15  44.0 gr.   2,645

    H380         47.0 gr.   2,624

    A2230        40.0 gr.   2,439

    A2460        41.5 gr.   2,474

    748          46.5 gr.   2,610

-----------------------------------------

200 grain JSP (Spitzer or Spire Point) or JRN

    IMR 4895     38.0 gr.   2,249 FPS

    IMR 3031     34.0 gr.   2,158

    748          43.0 gr.   2,435

-----------------------------------------

220 grain JSP (Spitzer or Spire Point) or JRN

    H4350*       46.0 gr.   2,369

* use magnum primers

-----------------------------------------

Discussion: IMR 3031 is a faster burning powder than IMR 4895, so you use less of it for similar volocities. I have listed powders of Hodgdon, Alliant and Accurate also. Use which ever you have handy. Slow burning powders generally work better in longer barrels and heavier bullets. 308 Cases will stretch, especially with hot loads, and you definitely need to check each in a case length guage and trim off excess brass. Of course, discard any cases that show a split anywhere on the case. For hunting at long distances with an unobstructed view, you should choose the spire point with a soft lead tip. For hunting at shorter ranges in brush, as is common in the woods of Alabama, you should choose a round nose bullet. They are less easily deflected and/or deformed by brush that you may be shooting through. The flat nose bullet is used especially in tubular fed magazines to prevent pressure on the primer in the cartridge above it. 

There are a number of other high performance bullets on the market. The "boattail" is a very good long range target match bullet. It has very good wind performance. If you are shooting for precision, you should weight each powder charge and put exactly the same amount in each case. Each case should be exactly on specs and the bullet should not be crimped into the case. (These bullets don't normally have a crimp ring anyhow.) (The Hunting bullets below DO have a crimp ring.) 
[image: image1.png]



Actually ALL 30 cal. bullets are .308 in diameter.


*
#2400, Reloader 7, 11, 12, and 15 are products of Alliant (formerly Hercules) Powders.
H380, H450, H4350 and H4895 are products of Hodgdon Powders.
IMR 3031 and IMR 4895 are products of IMR (formerly DuPont) Powder Company.
A#2230, and A#2460 are products of Accurate Arms Company
WMR, WSF, WSL, 760, 748, 680, 231 and 540 are products of Winchester, Components Div.

References: Accurate Arms Co. 1996 Reloading booklet - - Alliant Powder Co. Reloaders' Guide (1995) - - Hodgdon's Basic Reloaders Manual (1996) - - Speer Reloading Manual, #12 - - Winchester Reloading Components Manual, 14th edition - - Lyman Piston & Revolver Reloading Handbook, 2nd edition. - - VihtaVuori Oy Company, reloading booklet (1995) - - Numerous magazine articles on reloading special calibers and personal loads where noted. 

