Reloading The .38 Special 


Left to right:--148 gr. WC, 158 gr. SWC, 160 gr. SWC+GC, 
158 gr. RN, 158 gr. RN, 160 gr. JHP, 125 gr. JHP, 158 gr. JHP

(WC= WadCutter, SWC= Semi-WadCutter, HP= Hollow Point, RN= Round Nose), (JHP= Jacketed Hollow Point and if GC indicated, means Gas Check at bottom of lead bullet) 

Loads for .38 special:
(Note: These are near maximum loads (in some cases ARE MAXIMUM), you should use 10%. less to start.) Only if you see more than one load, is it less than maximum. If there is only ONE load, it is MAX. 
(See note on Powders below or read all about various Powders.) 
Bullets are .357" for jacketed and .358" for lead in diameter. Due to different barrel lengths, type of bullet, seating depth, primer type and other factors, you may not get near the FPS charted. It is just a guide and the reason you should start under these charges and work up. 

-----------------------------------------

90 grain Lead RNFP    Cowboy loads

    Titegroup  3.0 gr.   755  minimum

    Titegroup  3.5 gr.   969 Maximum

    Clays      2.5 gr.   705  minimum

    Clays      3.0 gr.   938 Maximum    

----------------------------------------

110 grain JHP (Jacketed Hollow Point)

    Bullseye   
4.5 gr.   1,085 FPS

    Unique     5.6 gr.   1,090

    Universal  5.6 gr.   1,143

    Clays      3.8 gr.     973

    HP38       5.2 gr.   1,043

    No. 2      5.6 gr.   1,083

    No. 5      7.3 gr.   1,090

    231 (+P)   5.7 gr.   1,015

    540 (+P)   8.7 gr.   1,070

-----------------------------------------

125 grain Lead RNFP or SWC

    Trail Boss 3.0 gr.   753 FPS minimum

    Trail Boss 5.3 gr.   952 FPS Maximum 

    Universal  4.3 gr.   872 FPS minimum

    Universal  4.7 gr.   1036 FPS MAXIMUM

    HP38       3.8 gr.   876  minimum

    HP38       4.8 gr.   1071 Maximum

    Titegroup  3.2 gr.   856  minimum

    Titegroup  3.8 gr.   985 Maximum

    Clays      2.5 gr.   810  minimum

    Clays      3.5 gr.   978 Maximum   

-----------------------------------------

125 grain JHP or JSP (Jacketed Hollow or Soft Point)

    Bullseye   4.4 gr.   1,000 FPS

    Unique     5.3 gr.   1,015

    Universal  5.2 gr.   1,019

    Clays      3.5 gr.     895

    HP38       4.8 gr.     914

    No. 2      5.3 gr.     990

    No. 5      6.8 gr.     860

    231 (+P)   5.3 gr.     935

    540 (+P)   8.2 gr.   1,000

-----------------------------------------

146-148 grain LWC (Lead Wad Cutter)

    Bullseye   2.8 gr.     815 FPS

    Unique     3.3 gr.     815

    Universal  3.8 gr.     940

    Clays      2.5 gr.     836

    HP38       3.8 gr.     879

    No. 2      4.2 gr.     ---

    No. 5      6.0 gr.     ---

    231        3.4 gr.     760

    540        5.2 gr.     785

    ZERO AMMO (commercial) 795 (reference only)

-----------------------------------------

146-150 grain JHP (Jacketed Hollow Point)

    Bullseye   3.8 gr.     850 FPS

    Unique     3.8 gr.     796 (mild) 

    Unique     5.1 gr.     860

    HP38       3.8 gr.     856

    No. 2      4.5 gr.     820

    No. 5      6.0 gr.     745

    231        3.4 gr.     820

    2400 (+P)  8.0 gr.     910 (+P) CAREFUL!

    Unique(+P) 5.5 gr.     964 (+P) CAREFUL!

-----------------------------------------

158 grain Lead RNFP or SWC COWBOY load

    Trail Boss 2.7 gr.   661 FPS minimum

    Trail Boss 4.2 gr.   804 FPS Maximum 

    Universal  3.5 gr.   756 FPS minimum

    Universal  4.5 gr.   974 FPS MAXIMUM

    HP38       3.1 gr.   782  minimum

    HP38       3.7 gr.   834 Maximum

    Titegroup  3.2 gr.   815  minimum

    Titegroup  3.8 gr.   920 Maximum

    Clays      2.8 gr.   812  minimum

    Clays      3.1 gr.   871 Maximum       

-----------------------------------------

158 grain LSWC or LRN (Semi-Wad, Rnd. Nose or flat point)

    Bullseye   3.6 gr.     810 FPS

    Unique     3.5 gr.     697 (mild load)

    Unique     4.3 gr.     920

    Unique     4.5 gr.     975 (actual in 7" bbl revolver)

    Unique     4.5 gr.     908 (5" bbl,powder UP)

       (Hi=930  Lo= 885.4   Spread=44.2 fps)

    Unique     4.5 gr.     797 (5" bbl,powder DOWN - see note)

       (Hi=879.5  Lo= 729.4   Spread=150.1 fps)

    Universal  3.5 gr.     756 (Cowboy load)

    Universal  4.5 gr.     974 

    Clays      2.8 gr.     812 (cowboy load)

    Clays      3.1 gr.     871

    HP38       3.1 gr.     782 (Cowboy load)

    HP38       3.7 gr.     834

    No. 2      4.0 gr.     756 (published load)

    No. 2      3.6 gr.     808 (in 7" bbl)

      hi=849  lo=728  ES=122 (powder up and down)

    No. 2      3.2 gr.     725 (in 7" bbl)

      hi=770  lo=679  ES=91 (powder up and down)

    No. 5      5.8 gr.     841 (published)

    No. 5      5.0 gr.     736 (in 7" bbl)

      hi=763  lo=688  ES=75 (powder up and down)

    No. 5      4.6 gr.     694 (in 7" bbl)

      hi=740  lo=648  ES=92 (powder up and down)

    231        3.1 gr.     753 (do not use less)

    231        4.5 gr.     830 (published MAX)

    231        4.0 gr.     898 (actual in 7" bbl revolver)

    WST        3.7 gr.     770 (MAX)

    WAP        4.5 gr.     681 (published)

    WAP        4.3 gr.     752 (in 7"bbl)*

      hi=821  lo=684  ES=137 (powder up and down)

    WAP        4.3 gr.     734 (in 4 5/8" bbl)*

      hi=815  lo=637  ES=178 (powder in normal positions)

    WAP        4.0 gr.     658 (in 7" bbl)*(NOT recommended)

      hi=714  lo=549  ES=165 (powder up and down)        

    540        6.8 gr.     880

    VV N320    3.9 gr.     872 (published)

    VV N320    3.2 gr.     723 (in 7" bbl)(same in 4 5/8" bbl)

      hi=758  lo=693  ES=65 (powder up and down)(DO NOT USE LESS)

    VV N340    4.8 gr.     882 

*WAP is not best choice for .38 special, too slow and inconsistent.

-----------------------------------------

158 grain Copper PLATED SWC

    W231       4.0 gr.     745 (published)

    W231       4.5 gr.     830 (published MAX)

    W231       4.0 gr.     811* (in 7" bbl revolver)

      hi=859  lo=743  ES=116 (powder up and down)

*(my personal medium load, Chrono'd myself)

-----------------------------------------

158-160 grain JHP or JSP (Jacketed Hollow or Soft Point)

    Bullseye   3.5 gr.     805 FPS

    Unique     4.2 gr.     800

    2400       7.6 gr.     850

    Universal  4.4 gr.     778

    HP38       3.7 gr.     819

    HS6        6.5 gr.     914

    No. 2      4.0 gr.     756

    No. 5      5.8 gr.     841

    231 (+P)   4.4 gr.     720*

    540 (+P)   6.6 gr.     800*

    2400 (+P)  7.8 gr.     910*

    Unique(+P) 4.4 gr.     885*

* All PLUS P loads for guns designed for it (20,000 PSI)

-----------------------------------------

180 grain LEAD (Truncated Cone) Flat Point

    (Cowboy loads - shot in 7 1/2" bbl.revolver - no need to reduce)

    VV N320    3.2 gr.     722 fps

      hi=780  lo=674   ES=105(powder up and down)

    WAP        4.3 gr.     773 fps *

      hi=875  lo=685   ES=190.2(powder up and down)

    WAP        4.0 gr.     639 fps *

      hi=710  lo=497   ES=213 (powder up and down)

    AA No.2    3.6 gr.     816 fps

      hi=857  lo=785   ES=72 (all shot level)

    AA No.2    3.2 gr.     722 fps

      hi=791  lo=625   ES=166 (Powder up and down)

    AA No.5    5.0 gr.     725 fps

      hi=785  lo=670   ES=115 (powder up and down)

    AA No. 5   4.6 gr.     690 fps

      hi=736  lo=647   ES=89 (powder up and down)

*WAP is not best choice for .38 special, too slow and inconsistent.

-----------------------------------------

180 grain JHP or JSP (Jacketed Hollow or Soft Point)

  ALL THESE LOADS ARE PLUS P, SPECIAL HOT - 21,000 PSI

    2400   (+P)7.5 gr.     895* FPS

    Unique (+P)4.4 gr.     875*

    HS-6   (+P)6.7 gr.     870*

    HS-7   (+P)7.6 gr.     885*

    H4227  (+P)11.0 gr.    890*

    IMR4227(+P)10.7 gr.    844*

    231    (+P)4.4 gr.     800*

* Use in PLUS-P Guns ONLY!!!!    

-----------------------------------------

200 grain LRN (Lead Round Nose)

    Bullseye   3.0 gr.     760 FPS

    Unique     3.6 gr.     780

    2400       7.0 gr.     870

    Universal  3.5 gr.     ---

    HP38       3.2 gr.     681

    HS6        5.0 gr.     794

-----------------------------------------

(Note (+P) means higher pressures, use in guns

designed for the +P cartridges.)

Note: More and more test are conducted with all rounds fired with powder UP against the primer for each shot, or DOWN on the bullet for each shot, then gently bring to horizontal to shoot. A significant difference in speeds is usually noted. It is always higher when powder is next to primer. For best accuracy in slow fire shooting, you may want to raise gun before every shot. 

Discussion: Bullseye is a faster burning powder and some people feel better suited for the .38 special. I prefer the slower burning Unique, and I also use it for .357 magnum, just more of it. It seems to burn completely in the .38 and is not as critical as the faster powders like Bullseye. Accurate's No. 2 and No. 5 are good choices. Winchester's 231 or WSL are also good powders for the .38 special. Winchester 231 is between Bullseye and Unique for the .38 special (see my personal load above). Hodgdon's Clays, International, Universal and HP38 are good choices. And Vihtavouri's N320 and N321 fall in this range for burning times. Even slower powders may provide more consistent speeds and will be tested in the future (No. 7, No. 9 and others like 2400 that are generally used for magnums). 

If you have a lightweight .38 or older gun, you may want to start with 10% less than max amounts listed above, and work up to the full loads. The "cowboy" loads would be good choices as listed, since these ARE reduced loads for that sport, where usually just hitting a metal plate to make a sound is all that is required of the round. 

Below, are three loaded rounds. Nickle cases look pretty but don't seem to take the number of resizings that plain brass cases do. Always make sure your lead bullets have been sized and lubricated. Note the dark band of lube on the 3 lead bullets. As you move up in loads, you at least need the gas check on the bottom of the bullet to prevent gas melting the lead and fouling the barrel. For any loads over 900-1,000 FPS, I suggest using a jacketed bullet. Jacketed bullets create a bit more wear on the rifling, but it's better than trying to wire brush out a bunch of lead build up. Just depends on what kinds of loads you plan to shoot. Normal, mild .38 loads work fine with lead bullets. A bullet with a copper gas check or copper plating is best for INDOOR ranges to keep lead vapors out of the air. 
[image: image1.png]





*
Bullseye, Unique and 2400 are products of Alliant (formerly Hercules) Powders.
Clays, Universal, HP38, HS6 and H4895 are products of Hodgdon Powders.
IMR 3031 and IMR 4895 are products of IMR (formerly DuPont) Powder Company.
No. 2, No. 5, #2230, and #2460 are products of Accurate Arms Company
WAP, WMR, WSF, WSL, 760, 748, 231 and 540 are products of Winchester, Components Div.
VV N320, N330 and N340 are products of VihtaVouri Oy Company

References: Accurate Arms Co. 1996 Reloading booklet - - Alliant Powder Co. Reloaders' Guide (1995) - - Hodgdon's Basic Reloaders Manual (1996) - - Speer Reloading Manual, #12 - - Winchester Reloading Components Manual, 14th edition - - Lyman Piston & Revolver Reloading Handbook, 2nd edition. - - VihtaVuori Oy Company, reloading booklet (1995) - - Numerous magazine articles on reloading special calibers and personal loads where noted. 

