Reloading The .45 ACP (Automatic Colt Pistol)


Left to right:-- 230 gr. RN, .45 with JRN, .45 with JHP, 230 gr. JRN

(WC= WadCutter, SWC= Semi-WadCutter,
HP= Hollow Point, RN= Round Nose), SP=Soft Point
(JHP= Jacketed Hollow Point and if GC indicated, means Gas Check at bottom of lead bullet) 

Loads for .45 ACP:

(Note: These are near MAXIMUM loads, you should use 10% less to start.)
(See note on Powders below or read all about various Powders.) 
Bullet sizes vary from .451" for jacketed to .452" for some lead in diameter. Due to different barrel lengths, type of bullet, seating depth, primer type and other factors, you may not get near the FPS charted. It is just a guide and the reason you should start under these charges and work up. 

----------------------------------------

155 grain LSWC (Lead, Semi-Wad Cutter)

    Bullseye   6.9 gr.   1,175 FPS

    Unique     7.8 gr.   1,190

    Universal  7.6 gr.   1,195

    Clays      5.2 gr.   1,082

    HP38       8.0 gr.   1,179 

    No. 2      7.5 gr.   1,204

    No. 5     10.0 gr.   1,189

    540        8.5 gr.     940

-----------------------------------------

185 grain JHP or JSP (Jacketed Hollow or Soft Point)

    Bullseye   5.3 gr.     914 FPS

    Unique     7.0 gr.     928

    Unique     6.0 gr.     765 (actual in Colt45 for mild load)

    Universal  7.2 gr.     993

    Clays      4.9 gr.     981

    HP38       5.9 gr.     906

    HS-6       8.6 gr.     917

    No. 2      7.5 gr.   1,077

    No. 5     10.2 gr.   1,102

    231        6.8 gr.     990

    540       10.0 gr.   1,015

    WST        5.6 gr.     935

    WAP        8.9 gr.   1,045

-----------------------------------------

200 grain LSWC (Lead, Semi-Wad Cutter)

    Bullseye   4.2 gr.     790 FPS

    Unique     5.1 gr.     810

    Universal  5.4 gr.     804

    Clays      4.3 gr.     888

    HP38       5.6 gr.     914

    HS-6       8.4 gr.     907

    No. 2      5.8 gr.     939

    No. 5      8.7 gr.   1,025

    231        5.5 gr.     910

    540        8.3 gr.     925

    WST        5.1 gr.     910

    WAP        7.6 gr.     970

-----------------------------------------

225 grain Speer JHP (Jacketed Hollow Point)

    Bullseye   4.8 gr.     884 FPS

    Unique     6.0 gr.     898

    Universal  6.2 gr.     820

    HP38       5.3 gr.     832

    HS-6       8.2 gr.     825

    No. 2      5.6 gr.     ---

    No. 5      8.5 gr.     ---

    231        5.2 gr.     824

    WAP        7.9 gr.     895

-----------------------------------------

230 grain LSWC or LRN (Semi-Wad Cutter or Round Nose)

    (these loads work for the RN Plated bullet,too)

    Bullseye   4.0 gr.     810 FPS (target accuracy load)

    Bullseye   4.8 gr.     884 hot load/Maximum.

    Unique     5.5 gr.     795  (I got 738 fps in my gun)

    Unique     5.5 gr.     913  (in Marlin auto carbine)

    Unique     6.0 gr.     840* (I got 777 fps in my gun)

    Unique     6.0 gr.     924* (in Marlin auto carbine)

    Universal  4.8 gr.     782

    No. 2      5.6 gr.     870

    No. 5      8.5 gr.     968

    231        5.1 gr.     870 (I got 734 in my S&W)

    540        7.6 gr.     860

    WST        4.5 gr.     805

    WAP        7.3 gr.     915

 * Alliant shows this a Maximum load

-----------------------------------------

230 grain FMJ or JHP (Jacketed Hollow Point)

    Bullseye   4.8 gr.     884 FPS

    Unique     6.4 gr.     880

    Universal  6.0 gr.     853

    HP38       5.3 gr.     832

    HS6        8.2 gr.     825

    No. 2      6.1 gr.     874

    No. 5      8.7 gr.     927

    231        5.1 gr.     785

    540        7.6 gr.     800

    WST        4.9 gr.     800

    WAP        7.4 gr.     885

-----------------------------------------

260 grain FMJ or JHP Revolver Loads

    Bullseye   4.5 gr.     725 FPS

    Unique     5.4 gr.     760

    HP38       5.1 gr.     800

    HS6        8.0 gr.     849

    No. 2      5.5 gr.     785

    No. 5      7.7 gr.     845

    231        5.0 gr.     788

-----------------------------------------

Discussion: The small case and short powder burning time of the .45 ACP requires a faster burning powder, like Bullseye or Unique. The almost standard bullet is the 230 grain round nose, since it feeds so well in autos. However, the SWC has a round enough nose that it feeds well in most autos and makes a cleaner target cut-out. If you were to be hunting or for protection, you might consider the Jacketed Hollow Point bullet shown in the loaded round. Whether you use the lead or full metal jacketed bullet is up to you, but the lead is, of course, much cheaper if you shoot very much. Because of lead fumes in the air, you should use a jacketed or copper plated bullet indoors. 
[image: image1.png]NI






*
Bullseye, Unique and 2400 are products of Alliant (formerly Hercules) Powders.
Clays, Universal, HP38, HS6 and H4895 are products of Hodgdon Powders.
IMR 3031 and IMR 4895 are products of IMR (formerly DuPont) Powder Company.
No. 2, No. 5, #2230, and #2460 are products of Accurate Arms Company
WMR, WSF, WSL, 760, 748, 231 and 540 are products of Winchester, Components Div.

References: Accurate Arms Co. 1996 Reloading booklet - - Alliant Powder Co. Reloaders' Guide (1995) - - Hodgdon's Basic Reloaders Manual (1996) - - Speer Reloading Manual, #12 - - Winchester Reloading Components Manual, 14th edition - - Lyman Piston & Revolver Reloading Handbook, 2nd edition. - - VihtaVuori Oy Company, reloading booklet (1995) - - Numerous magazine articles on reloading special calibers and personal loads where noted. 

